Q:2

Full Factorial Design

Factors:   2   Base Design:         2, 4

Runs:     12   Replicates:             3

Blocks:    1   Center pts (total):     0

Factorial Regression: Yield versus Temperature, Pressure

H01: All row treatment (levels of temperature) is equal. (H01: αi = 0; i= 1,2)

H02: All column treatment (level of pressure) is equal. (H02: βj = 0; j= 1,2)

H12: All row and column treatment interaction effects are equal.

(H12: γij = 0; i=j = 1,2)

OR

H01: All row treatment (levels of temperature) is not significant.

(H01: αi = 0; i= 1,2)

H02: All column treatment (level of pressure) is not significant.

(H02: βj = 0; j= 1,2)

H12: All row and column treatment interaction effects are not significant.

(H12: γij = 0; i=j = 1,2)

Analysis of Variance

Source                    DF    Adj SS    Adj MS  F-Value  P-Value

Model                      3  0.449167  0.149722     6.20    0.018

Linear                   2  0.441667  0.220833     9.14    0.009

Temperature            1  0.140833  0.140833     5.83    0.042

Pressure               1  0.300833  0.300833    12.45    0.008

2-Way Interactions       1  0.007500  0.007500     0.31    0.593

Temperature*Pressure   1  0.007500  0.007500     0.31    0.593

Error                      8  0.193333  0.024167

Total                     11  0.642500

Conclusion:

From the ANOVA table, the p-value for Temperature and pressure is smaller than the α-value (α= 0.05), the hypothesis is rejected. that is, the temperature and pressure levels are significant. Since p-value for interaction for temperature and pressure is larger than the α-value (α= 0.05), it is concluded that there is no interaction effect between temperature and pressure.

Model Summary

S    R-sq  R-sq(adj)  R-sq(pred)

0.155456  69.91%     58.63%      32.30%

Coded Coefficients

Term                   Effect     Coef  SE Coef  T-Value  P-Value   VIF

Constant                       90.5250   0.0449  2017.21    0.000

Temperature            0.2167   0.1083   0.0449     2.41    0.042  1.00

Pressure               0.3167   0.1583   0.0449     3.53    0.008  1.00

Temperature*Pressure  -0.0500  -0.0250   0.0449    -0.56    0.593  1.00

Regression Equation in Uncoded Units

Yield = 89.317 + 0.00367 Temperature + 0.00517 Pressure - 0.000010 Temperature*Pressure

A regression model based on the coded observation is as follows:

Y = 90.525 + 0.1083 x1 + 0.3167 x2

The equivalent uncoded model based on the original data can be obtained as follows:

Y = 89.317 + 0.00367 x1 + 0.00517 x2

The residual analysis does not show any kind of unusual observations. The residuals are normal, and the run orders are also random.

Q:3

Full Factorial Design

Factors:   3   Base Design:         3, 8

Runs:     16   Replicates:             2

Blocks:    1   Center pts (total):     0

Factorial Regression: Firing Time versus Factor A, Factor B, Factor C

H01: levels of Factor A is not significant.

(H01: αi = 0; i= 1,2)

H02: level of Factor B is not significant.

(H02: βj = 0; j= 1,2)

H03: level of Factor C is not significant.

(H03: µk = 0; k= 1,2)

H123: All row and column treatment interaction effects are not significant.

(H12: γijk = 0; i=j=k= 1,2)

Analysis of Variance

Source                          DF   Adj SS   Adj MS  F-Value  P-Value

Model                            7  1236.94  176.705    36.72    0.000

Linear                         3  1146.69  382.229    79.42    0.000

Factor A                     1   588.06  588.063   122.19    0.000

Factor B                     1   473.06  473.063    98.30    0.000

Factor C                     1    85.56   85.562    17.78    0.003

2-Way Interactions             3    90.19   30.062     6.25    0.017

Factor A*Factor B            1    76.56   76.562    15.91    0.004

Factor A*Factor C            1     3.06    3.063     0.64    0.448

Factor B*Factor C            1    10.56   10.563     2.19    0.177

3-Way Interactions             1     0.06    0.063     0.01    0.912

Factor A*Factor B*Factor C   1     0.06    0.063     0.01    0.912

Error                            8    38.50    4.813

Total                           15  1275.44

Model Summary

S    R-sq  R-sq(adj)  R-sq(pred)

2.19374  96.98%     94.34%      87.93%

Coded Coefficients

Term                         Effect    Coef  SE Coef  T-Value  P-Value   VIF

Constant                             26.688    0.548    48.66    0.000

Factor A                    -12.125  -6.062    0.548   -11.05    0.000  1.00

Factor B                    -10.875  -5.438    0.548    -9.91    0.000  1.00

Factor C                     -4.625  -2.313    0.548    -4.22    0.003  1.00

Factor A*Factor B            -4.375  -2.188    0.548    -3.99    0.004  1.00

Factor A*Factor C             0.875   0.438    0.548     0.80    0.448  1.00

Factor B*Factor C             1.625   0.813    0.548     1.48    0.177  1.00

Factor A*Factor B*Factor C    0.125   0.063    0.548     0.11    0.912  1.00

Regression Equation in Uncoded Units

Firing Time = 79.4 + 4.4 Factor A - 3.475 Factor B - 0.001594 Factor C

- 0.975 Factor A*Factor B + 0.000031 Factor A*Factor C

+ 0.000081 Factor B*Factor C + 0.000006 Factor A*Factor B*Factor C

A regression model based on the coded observation is as follows:

Y = 26.688 -6.062 x1 – 5.428 x2 – 2.313 x3 -2.188 x12

The residual analysis does not show any kind of unusual observations. The residuals are normal, and the run orders are also random.